MAGNETIC RESONANCE IMAGING of LABORATORY ANIMALS LUNGS USING HYPERPOLARIZED 129XE
Ya.V. Fattakhov 1, A.R. Fakhrutdinov 1, D.A. Konovalov 1, V.L. Odivanov 1, V.A. Shagalov 1, A.A. Bayazitov 1, F.V. Bashirov 2, V.A. Markosyan 2, 
V.M. Vodovozov 3, A.V. Pavlenko 3, R.F. Kurunov 3, V.M. Baev 3, 
M.V. Kuleshov 3, V.V. Eremkin 3, A.A. Baturina 3, M.S. Sirko 3
1 - Zavoisky Physical-Technical Institute, FRC Kazan Scientific Center of RAS, 10/7 Sibirskiy Trakt str., Kazan, 420029, Russia

2 - Federal State Budgetary Educational Institution of Higher Education Kazan State Medical University of the Ministry of Health of the Russian Federation, 420012, 49 Butlerova St., Kazan, Russia

3 - Joint-Stock Company "D.V. Efremov Scientific Research Institute of Electrophysical Equipment", 196641, village Metallostroy, the road to Metallostroy, 3, Saint Petersburg, Russia
The purpose of this work was to develop a technique for obtaining NMR images of plants and laboratory animals using hyperpolarized (HP) 129Xe. An HP 129Xe production facility was created [1], and the MRI system [2] was modified to run on xenon atoms. A device for monitoring the respiration of laboratory animals has also been developed and manufactured, which allows periodic switching of the supply of a breathing mixture or xenon gas, as well as ensuring respiratory retention during measurement. The technology of tracheostomy of laboratory animals has been developed to ensure their respiration during lung scanning. To obtain images on samples with hyperpolarized gas, the pulse program "Fast gradient Echo" was developed, based on the use of exciting pulses with small rotation angles of the magnetization vector (in our case ~9( ), which allows performing a full image scanning cycle with one portion of gas. An experiment was conducted to obtain images of laboratory animals' lungs using HP 129Xe.
1.In Russian. V.M. Vodovozov, A.V. Pavlenko, R.F. Kurunov, V.M. Baev, M.V. Kuleshov, V.A. Chumichev, V.V. Eremkin, A.A. Baturina. An experimental sample of a laser installation for the development of hyperpolarized 129Xe /. – International conference "XXV Kharitonov thematic scientific readings. Modern Laser Technologies", Sarov, March 25 – 29, 2024, FSUE RFNC-VNIIEF, Abstracts.

2.Fattakhov Ya. V., Anikin A. N., Bayazitov A. A., Fakhrutdinov A. R., Khabipov R. Sh., Odivanov V. L., Salikhov K. M., Shagalov V. A., Reshetnikov N. M., Abdulganieva D. I., Mikhailov M. K., Ryzhkin S. A., Anisimov V. I. The First Results of Using a Specialized MRI System with Magnetic Field of 0.4 T // Modern Development of Magnetic Resonance, Kazan, September 28–October 2, 2020. Book of Abstracts, P.184.
